CD22-35-485

LCD SCREEN (Black connector on Proto)

| LCD CONNECTION | WIRE COLOR | SHIELD CONNECTION | PROTO NUMBER (LEFT,1 – RIGHT,8) |
| --- | --- | --- | --- |
| GND | YELLOW | GND | 7 |
| VCC | ORANGE | 5V | 8 |
| SDA | PURPLE | A4 | 6 |
| SCL | GREY | A5 | 5 |

SWITCH

| SWITCH CONNECTION | WIRE COLOR | SHIELD CONNECTION | PROTO NUMBER (LEFT,1 – RIGHT,8) |
| --- | --- | --- | --- |
| POLE | ORANGE | D5 | 4 |
| LED - | BLACK | GND | 7 |
| LED + | RED | D6 (220-ohm resistor) | 5 |
| THROW | ORANGE | GND | 6 |

JOYSTICK

| JOYSTICK CONNECTION | WIRE COLOR | SHIELD CONNECTION | PROTO NUMBER (LEFT,1 – RIGHT,8) |
| --- | --- | --- | --- |
| JOYSTICK X | BROWN | A0 | 5 |
| JOYSTICK Y | ORANGE | A1 | 4 |
| 5V | WHITE | 5V | 8 |
| ENCODER A | BLUE | D2 (2.2 k-ohm resistor) | 1 |
| ENCODER B | GREEN | D3 (2.2 k-ohm resistor) | 2 |
| SWITCH | YELLOW | D4 | 3 |
| SWITCH | BLACK | GND | 7 |
| GROUND | RED | GND | 7 |

RS485

| SWITCH CONNECTION | WIRE COLOR | SHIELD CONNECTION | PROTO NUMBER (LEFT,1 – RIGHT,8) |
| --- | --- | --- | --- |
| 12V | BROWN | VIN | 6 |
| GND | BLUE | GND | 7 |
| B (FERRULE) | WHITE | B (SCREW) | B (SCREW) |
| A (FERRULE) | BLACK | A (SCREW) | A (SCREW) |